1.   a)   UNDERGROUND JELLY FILLED CABLES  (FOR OUTDOOR WIRING)

0.51mm annealed copper conductor, polythene insulated cores variously colour coded, twisted into pairs, pair laid up,  fully filled with petroleum Jelly filling compound, cores wrapped with non-hygo scopic tape under moisture  barrier pokly aluminium laminated foil tape, bedded with water proofed polythene material and outer jacketed with high quality polythene black compound, armoured Telephone Cable confirming  to ITD Specification S/W-143B.

i) 20 pair ii) 50 pair iii) 100 pair

(Branded makes like Birla Ericsson , Vindia Telelink, Finolex, BML etc.  all TEC approved makes only)

b)  INTERNAL SWITCH BOARD PVC CABLE (FOR INDOOR WIRING)

0.51mm annealed tinned electrolytic copper conductor insulated and sheathed with PVC compound as per BS: 6746 Specification, twisted pairs bunched together in concentric layers and wrapped with PVC/ melinex tape, as per ITD-113C Specification, Telephone Cable.

i) 2 pair ii) 4 pair iii) 10 pair

(Branded makes like Birla Ericsson , Vindia Telelink, Finolex, BML etc.  all TEC approved makes only)

2.    a)  Krones for MDF with mounting base

b)  Internal protection module (IPM) against high voltages

3.    Analog Telephones

Analog telephone with Key padlock, Ringer volume control, Tone/ Pulse switchable, Redial, Mute, Flash, pulse, Desk/Wall mountable, Last number redial capacity.(TEC approved branded products like Siemens, Beetel, BPL, CG etc. only)

4.    Fiber Optic Converter and Switches

a) 12 Port LIU

Fiber optic patch panel	- 19-inch, Rack mounted Fiber optic patch panel

Height				- 2 U, 3.5 inches

Number of fibers		- 6, 12, 24 or 48

Number of OSP Cables 	- Minimum 2

for termination

Grounding			- 2 Nos. of earthing lugs, pre-loaded

Cable Management rings	- Front and rear cable management rings, pre-loaded

Number of 6-port / 12-port 	- 4/4 Max.

adapter plates

b) SC Connector

Operating temperature	- 40 Degree C to +85 Degree C

Durability & colour

MM connector	 		- 500 cycles, beige

SM connector			- 220 cycles, blue

Ferrules			- Pre-radiused Ceramic Ferrules

Attenuation			- Not more than 0.75 dB per mated pair

c)  SC-SC Patch cord		- 3 meter patch cord with SC interface in both ends

d)  Switch				- Should support 24 ports 10/100 with 2 10/100/100

SFP links