10. Piezoresistive Pressure Transducer (0-20 bar)

Absolute pressure transducer for measuring static and dynamic absolute pressure.

Range 				-	0 –20 bar (abs)

Sensitivity          			-	25 mV/bar

Linearity				-	+ 0.3% FSO

Overload				-	50 bar

Operating  temperature range	-	20-120 o C

Construction			-	Flush diaphragm

Reference				-	Absolute zero

Excitation				-	Constant current of 2-5 mA

Volume Change                  	-            <0.2 mm3

Shock Resistance                    	-           1000 g

Connector                               	-           Fischer Type SE 102A053