1

1. Series Impedance Element

[A	B    =[ 1	Z

C	D]      0	1]

2. SHUNT ADMITTANCE Element

[A	B    =[ 1	0

C	D]      y	1]

3. LOSSLESS TRANSMISSION LINE

[A	B    =[ cosβl	  jZ0sinβl

C	D]      jsinβl/Z0  cosβl    ]

2

4. N:1 TRANSFORMER

[A	B    =[ N	  0

C	D]      0      1/N ]

S-Parmeters of Lossless Transmission Line

SINCE THE TRANSMISION LINE IS LOSSLESS, α=0

THEREFORE

[S]= [ O  	         	exp(-jβl)

exp(-jβl)   	0	      ]

3

This structure can be implemented in stripline as

(Main inductance with side capacitances)

Where

jZ0sinβl=jwL

4

This is realized in stripline as

(Main capacitance with side inductances)

Where

jZ0sinβl=1/(jwC)

From the above two equations we can find out the length l.

For  inductance element, we use higher impedance (Z0 = 100Ω for stripline)

For capacitance element, we use lower impedance (Z0 = 20Ω for stripline)

Hence, for any inductor or capacitor, the width stays same, only length varies depending on L or C values.

Put  jTanβl=s

Then the short circuit and open circuit impedances become

Zsc=jZ0tanβl=Z0s

Zoc=Z0/jTanβl=Z0/s

5

When this is compared with the reactance of an inductor and susceptance of a capacitor respectively, it’s  seen that an inductor L can be represented by a short circuited stub and a capacitor C can be represented by an open circuited stub as shown below.

Here the length of the stub is always the same as shown but the width w varies depending on the L or C value.

ABCD PARAMETERS IN GENERAL

For reciprocal network,	AD – BC = 1

If a network is symmetrical,		A = D

For a lossless network,	 A and D are real

B and C are imaginary

6

Zi1 = √AB/CD

Zi2 = √BD/AC

Zinput = (AZL + B)/(CZL + D)

RELATIONSHIP OF ABCD WITH

S-PARAMETERS

S11 = (A + B/Z0 – CZ0 – D)/∆

S12 = 2(AD – BC)/∆

S21 = 2/∆

S22 = ( -A + B/Z0 – CZ0 + D)/∆

Where,	∆ = A + B/Z0 + CZ0 + D

FILTER REALISATION

BUTTERWORTH FILTER OR MAXIMALLY FLAT FILTER

L (dB) = 10log (1 + ω2n )

La = 3 dB for a butterworth response

gn values for a Butterworth Response:

g0 = 1

g1 = 2 sin [ (2k-1)/2n ]                ; k=1,2,3….n

gn+1 = 1

7

CHEBYCHEV FILTER OR EQUIRIPPLE FILTER

A = 10log [1 + (10Am/10 – 1) cos2 (ncos-1ω’) ]

Where,

n = order of the filter

Am = Ripple Magnitude in dB

ω’ = Bandwidth over which the insertion loss has maximum ripple

Here,

ω’ = ω/ω0                      for a Low Pass Filter

ω’ = Q(ω/ω0 - ω0/ω)   for a Bandpass Filter

ω’ = - ω0/ω                for a High Pass Filter

gn values for  Chebychev Response:

g0 = 1

g1 = 2a1/γ

gk = 4akak-1/bk-1gk-1       ; k=2,3,4….n

gn+1 = 1 ; n-odd

gn+1 = tanh2(β/4) ; n-even

where,

ak = sin{(2k-1)π/2n}     ; k=1,2….n

bk = γ2 + sin2(kπ/n)      ; k=1,2,…n

β = ln[coth Am/17.37]

γ = sinh (β/2n)

8

STATIC Analysis

Static analysis produces  T.L. (Transmission Lines ) parameters which are frequency independent.

Zo = (L/C)½

where L is the inductance / unit length of the transmission line and

C is the capacitance/ unit length of the transmission line .

Zo= (La Ca / C Ca) ½

where La is the inductance / unit length of the transmission line and

Ca is the capacitance/ unit length of the transmission line , when the dielectrics are replaced by air i.e. €r =1.

Zo= 1/c(C Ca) ½

[ Inductance/unit length does not depend on the surrounding substrate. ]

where c= velocity of light in free space.

The phase velocity np of the QUASI-TEM wave propagating along the transmission line is given as

np= c/(eff)½

eff = c2 / np2 = C /Ca

Wavelength  λ= λ o /(eff)½

LAPLACE  EQUATIONS are to be solved for C calculation.

TO DETERMINE STATIC PARAMETERS , we only need to calculate

C/unit length of Transmission Line with and without SUBSTRATE.

9

Hybrid Ring Coupler

Scattering Matrix characterizing the matched hybrid ring is given by

0	   	-jYb/Yc	     0			jYa/Yc

-jYb/Yc 	    0	   	    -jYa/Yc		0

0	   	-jYa/Yc	      0			-jYb/Yc

jYa/Yc	    0    	     -jYb/Yc		0

(Ya/Yc)2+(Yb/Yc)2=1,

Scattering Matrix of the rat-race hybrid is given by

0	   -j/√2	     0		j/√2

-j/√2 	    0	   	    -j/√2	0

0	   -j/√2	      0		-j/√2

j/√2	    0    	     -j/√2	0

10

Two-Stub Branch Line Coupler

For S11=0,	BYc=C/Yc

Mid-Band Parameters:

S11=S14=0,		S12=-jYc/Ya,	S13=-jYb/Ya

Yc2=Ya2-Yb2

For 10dB,900 Coupler

S12=-j3/√10,	S13=-1/√10

Ya=√10Yc/3,	Yb=Yc/3

11

Two-way Power divider

Za=√2Zc

Scattering Matrix can be written as

0		1/√2		1/√2

1/√2		S22		-S22

1/√2		-S22		S22

|S22|=1/2

Matched Power Divider

12

Overall Scattering matrix of the Matched Power divider is

0	     -j/√2	     -j/√2

-j/√2		0		0

-j/√2		0		0

Unequal Power Divider

Design forumulas for this power divider are as follows

P3/P2=K2

Z1=Zc(K/1+K2)1/4

Z2=ZcK3/4(1+K2)1/4

Z3=Zc(1+K2)1/4/K5/4

Z4=Zc√K

Z5=Zc/√K

R=Zc(1+K2/K)

NEXT