1.   Lathe - 5 Nos.

Specifications:

Height of centres                      :        175 mm

Swing over bed                         :        350 mm

Distance between centres       :        800 mm

Movement of cross slide          :         180 mm

Spindle bore                           :           40 mm

Spindle diameter                    :           50 mm

Range of speeds                    :         100 – 1200 rpm

Feed range

Longitudinal               :       0.05 – 2 mm/rev

Transverse              :       0.02 – 1.2 mm/rev

2. Radial drilling machine – 1 No.

Specifications:

Drilling capacity in steel   	 :      50 -- 70 mm

Drilling head spindle nose taper	 :    MT5

Maximum spindle stroke 	:    350 -- 450 mm

No.  of spindle speeds   	:    12  --  16 Nos.

Range of spindle speeds 	:   100 -- 1600 rpm

Minimum / Maximum drilling radius	:    510/1820 mm

Column diameter 	:    360 mm

Working surface	:   1800 x 1000 mm

Vertical traverse of arm 	 :   900 mm